# Lab 3: Introduction to the e-puck Robot

Q2 (5):	Type N <Enter> in your minicom terminal. Then put a finger in front of one of the IR sensors and type N <Enter> again. What are you measuring? When was the value higher?

The amount of reflected light. When you put the finger in front of the sensor the value should be higher as you are measuring the light reflected by your finger. The measured values should be between 0 – 100 in case there is nothing in front of the sensors, and 1000-4000 if there is something right in front of it.

Q3 (5):	Put different objects (white sheet of paper, dark objects …) at the same distance in front of the sensor and take a measurement each. Explain why the values are different, even though the distance is the same.

The value indicates the amount of reflected light which is measured. This value depends on the distance BUT ALSO on the reflectivity of the material.

Q4 (10):	Give a sequence of commands such that the robot would move along the following line (approximately). 

d,200,100
s
d,200,200
s
d,200,-200
s
d,200,200
s

S5:	Stop the motors and execute the following commands

p,0,0

d,200,-200

While the e-puck is moving, press Q <Enter> from time to time.

Q6 (5):	What do the values returned by Q mean? By how much does Q increase per second?

p sets the internal step counter for each motor, p,0,0 sets the step counter for both stepper motors to 0. Q displays the internal step counter. The step counter increases by 200 steps per second.

Q7 (5):	Stop the motors again and execute the following commands

The rotational speed, defined as the angular distance traveled per time, for the center of robot, ωc, is computed as:

ωc=ΔθΔT=2πT=2π25.62

Rc=15.9cm

All components in the robot will have the same rotational speed while travelling the circle. Their linear speed however will depend on the distance from the center of rotation, the farther they are, the faster they should go to cover the same angle.

ω=v/R

For the two wheels we have:

a=5.3cm

R1= Rc-a/2

R2= Rc+a/2

The linear speeds can then be written as:

v1=R1 ω=15.9-5.322π25.62=3.25cm/s

v2=R2ω=15.9+5.322π25.62=4.55cm/s

The transformation from linear speeds to proper commands is done as below:

u1=v1P*S=3.2513*1000=250

u2=v2P*S=4.5513*1000=350

Where P is the wheel perimeter and S is the number of steps per wheel rotation, as provided.

S8:	Execute the computed command.

Q9 (5): Try to increase the wheel speeds such that your e-puck travels the same circle faster.

Traveling the circle faster means to cover the same angle in less time, thus higher ω.

To stay on the same circle, the rotation radius should be kept the same as the previous part.

S10:	Execute the computed commands and observe what happens when you try to travel too fast and try to explain it.

There is a maximum speed the wheels can rotate on. 1000 steps per second. Reaching the maximum speed on the wheel on the outer side first, the robot will travel on a circle with a bigger radius since R=v1+v2v1-v2*d2. Reaching the maximum speed on both wheels the robot will travel on a straight line.

You can now quit minicom by typing Ctrl-A Q.

## Obstacle Avoidance

In the previous lab, we introduced the Braitenberg robot controller. In this section, you will run the Braitenberg obstacle avoidance algorithm on the e-puck itself.

S11:	Within your lab03 directory, type the following commands to program your robot with the Braitenberg obstacle avoidance program:

cd obstacleavoidance/

epuckupload -f obstacleavoidance.hex 999

Don’t forget to press the blue reset button on the e-puck as soon as the dots appear on the screen (stars indicate that program is being uploaded). Now, put the robot in the arena and observe it doing obstacle avoidance.

S12:	Do the same for obstacleavoidance_log. This code differs in three ways from the previous algorithm:

- The output of the infrared sensors is passed through a log function. Since the infrared sensors output decreases exponentially with the distance, this linearizes the Braitenberg inputs.

- An average over 10 samples is computed in order to reduce measurement noise.

- The sensors are calibrated by taking a series of measurements at the beginning. Therefore, make sure that there is no obstacle around when you switch on (or reset) the e-puck.

Q13(5):	What differences in obstacle avoidance behavior do you observe between the two algorithms?

As the output of the sensors is linearized and smoothed the obstacle avoidance should be smoother.

Q14(5):	Important: in the Makefile, make sure to write the correct path to your e-puck library. Now open obstacleavoidance_log/main.c and modify the algorithm such that it averages over 1000 instead of 10 samples. Recompile obstacle-avoidance_log.hex by typing make in that folder and upload it. Does this improve the robot’s behavior? Explain what happens.

Because the code now averages 1000 samples instead of 10 it takes a lot longer for the augmented sensor readings to indicate an obstacle and the epuck will now tend to run into obstacles before starting an avoidance maneuver.

I15(10):	There are also other ways of programming an obstacle avoidance controller. Write a simple rule-based obstacle avoidance algorithm, based on the skeleton provided in rulebased/main.c. Once you have finished programming, compile and upload your program, and test it on the robot.

Many possibilities exist. No specific solution.

Q16(10): 	In your opinion, what are the advantages and disadvantages of the Braitenberg controller with respect to the rule-based controller? (If you have time, test both your obstacle avoidance algorithms in a U-obstacle (like you saw in the previous lab. You can create a U-obstacle using extra white walls.)

Rulebased is very simple to implement. That is however probably the only advantage since the behavior of the Braitenberg controller is much smoother.

The Braitenberg principle can also be used to program an object following algorithm: instead of avoiding obstacles, the e-puck is supposed to follow an object (e.g. your hand) in front of it.

B17(10):	Find a simple way of modifying the Braitenberg obstacle avoidance algorithms to produce object following behavior. We prepared the folder objectfollowing for you with a copy of the obstacleavoidance program.

Make the necessary modifications in objectfollowing, compile (make), upload (epuckupload -f objectfollowing.hex 999) and test your algorithm.

Change the sign of the coefficients simply, by multiplying by -1.

## Dead-reckoning navigation

This exercise will demonstrate dead-reckoning on the e-puck and the importance of calibration.

I18(5):	Open the file drnavigation/main.c and try to understand how the code works. There are three functions which are relevant to the dead-reckoning navigation. e_set_dr_position() sets the position at which the robot thinks it is. By calling e_set_dr_position(0,0,0) the robot thinks it is at the origin of a local coordinate system pointing into the direction of the y-axis (=”up”=”North”). e_do_dr() reads the number of steps each motor took since the last call of e_do_dr() and computes the new dead-reckoned position based on the previous position (determined during the last call of e_do_dr()) and the motor steps. e_get_dr_position(&dr_x,&dr_y,&dr_theta) retrieves the actual dead-reckoned position. First the motor speed is set to 300 for both motors so that the e-puck goes forward. Then the dead-reckoned position is continuously updated e_do_dr()and read until the robot has traveled the desired distance. Similarly the robot turns 180 degrees, travels the same distance in the opposite direction and turns another 180 degrees. At the end of the program’s execution the robot should be in the starting position and orientation (=”pose”).

I19(5):	Make sure the robot has enough space to travel 20 cm or more forward. Using a pencil mark the starting position on the table. Compile the program, upload and execute it.

Q20(10):Notice how the robot does not really return very well to the starting position? This is because the distance between the wheels and steps traveled per wheel rotation are not properly calibrated. Hint: you can compare the distance which the robot traveled by comparing it to the short side of an A4 paper which is about 20 cm.

Open the file EpuckDevelopmentTree/library/motor_led/e_motors.h. In line 56 and 57 you will find the two #define statements

#define STEPS_PER_M 4443

#define WHEEL_DISTANCE 0.060

where the two relevant values are defined. Looking at how the e-puck went compared to how it should have and looking at the effect of the wheel distance on the dead-reckoning computation (lecture notes), which of the following set of #define statements probably leads to the best result? Try to solve it by analyzing the dependencies between the STEPS_PER_M and WHEEL_DISTANCE value and the dead-reckoned distance traveled and angle turned. Verify it by modifying the code accordingly, recompiling and running the robot. Remember that every time you modify something in EpuckDevelopmentTree/library/motor_led/e_motors.h you need to rebuild the library by going into EpuckDevelopmentTree/library/ and execute make clean and then make. Do this before recompile drnavigation/main.c.

#define STEPS_PER_M 10237

#define WHEEL_DISTANCE 0.053

b.)

#define STEPS_PER_M 7767

#define WHEEL_DISTANCE 0.053

c.)

#define STEPS_PER_M 7767

#define WHEEL_DISTANCE 0.060

Solution b leads to the best results. If the STEPS_PER_M  value is too small the robot will not travel far enough (less than 20cm). If WHEEL_DISTANCE is too small the robot does not turn far enough, i.e. it turns too slowly.

## Feature-based navigation

When dead reckoning is properly calibrated, the e-puck is able to return to its initial position really well however after a while it will have drifted quite a bit. One way to make sure that we get back to the right position every time is by using very basic featured-based navigation.

The following experiment will demonstrate dead-reckoning with and without correction through feature observation. In order to better observe the effect of the drift we put the robot in a corridor and let it use its infrared sensors to not touch the walls. This way the robot will not drift in the cross-corridor direction and we can better observe the drift in the along-corridor direction.

I21(5):	Set up the e-puck and three walls as shown in the figure below. The e-puck should have about 5 mm distance to each wall with the front of the robot (where the camera is) pointing to the exit of the corridor.

First you need to make sure that your dead-reckoning is properly calibrated. Open the file EpuckDevelopmentTree/library/motor_led/e_motors.h and set line 56 and 57 to

#define STEPS_PER_M 7767

#define WHEEL_DISTANCE 0.053

Make sure you recompile the library by executing make and then make clean in EpuckDevelopmentTree/library/. Compile and upload the file featurenavigation/main.c. This code will make the e-puck go forward for 20 cm then turn 180 degrees go forward 20 cm again and turn another 180 degrees. After uploading the program, push the RESET button. This will make the program start. At the beginning of the program there is a 5 second pause before the robot starts. This will give you enough time to properly place the robot in the corridor.

After the robot starts it will execute the sequence (go 20 cm→ turn 180° → go 20 cm→ turn 180°) infinitely many times, but after a while enough dead reckoning error will have accumulated such that the robot will start pushing the back wall. Let the robot go back and forth a few times and observe the drift.

I22(5):	Now we want to use the back wall as a feature each time we are next to it. Open the file featurenavigation/main.c. Look for the function align(ref_point),  remove the comments (“//”) , recompile and re-upload. Now set the robot in the corridor again, just as in the previous figure.  The robot will go back and forth again, but every time after the second turn, when the robot is near its initial position, it will measure the distance to the wall (i.e. the feature) and adjust its position such that it is the same as at the beginning. It will then reset its internal dead-reckoning status.